# Design and implementation of Earthquake Detector

Lovely professional university

Jalandhar, Punjab,

India

- Introduction

Earthquakes are one of the most devastating natural disasters that cause significant damage to infrastructure and result in the loss of lives. Timely detection and alerts can help reduce the damage and improve emergency response. This paper introduces a low-cost earthquake detector using sensors, microcontrollers, and real-time notification mechanisms to aid in preparedness and safety

2. Methodology

The earthquake detector system is designed using an accelerometer to detect ground motion and a microcontroller for processing the data. Upon detecting significant vibrations, the system activates an alarm and sends alerts through a connected module. The methodology includes hardware integration, signal processing, and real-time response mechanism.

3. Results and Discussion

The proposed earthquake detector was tested under controlled vibrational stimuli. It successfully detected threshold-level tremors and activated alerts within seconds. The system is cost-effective and suitable for deployment in schools, homes, and workplaces. Further enhancements can include cloud connectivity and integration with emergency services.

Fig 1 Earthquake detector using ardunio

4. earthquake detector structure

⬆ Sensors
    - Accelerometer (e.g., ADXL335 or MPU6050) to detect vibrations or motion in X, Y, Z directions.

⬆ ADC (Analog to Digital Converter) and Sensor Circuit
    - If using analog accelerometer: Signal converted to digital via Arduino's in-built ADC.
    - Proper connections with voltage dividers or filters if needed.

⬆ Processor
    - Arduino Uno/Nano used to process input from the accelerometer.
    - Compares vibration data against threshold values.

⬆ Driver
    - Transistor (e.g., BC547 or 2N2222) used to drive the output device (buzzer, LED, GSM module, etc.).

⬆ Actuators (Optional)
    - In case of mobile alert system: motors or indicators can be triggered.
    - Can activate relays for alarms or notifications.

⬆ Chassis and Body Structure
    - Compact platform to hold Arduino, accelerometer, power module, and output system.
    - Should be vibration-isolated or mounted firmly.

⬆ Power Supply (5V / 12V DC)
    - 5V for Arduino and sensors.
    - 12V for relays, GSM modules, or larger actuators (if used).
    - Can use adapter, battery + regulator (7805), or USB power.

Fig 2 Block diagram

5 components and price

- Arduino with USB cable

- Adxl335 module 3 axis analog output accelometer

- Buzzer

- BC547 transistor

- I k ohm resistor

- Breadboard

6 use and functionality of each component

- Ardunio

The Arduino acts as the brain of the system. It continuously reads the analog signals from the ADXL335 accelerometer, which detects vibrations or movements on the X, Y, and Z axes. The Arduino processes this data in real-time and checks if the vibration level crosses a pre-set threshold that indicates an earthquake-like motion. If it does, the Arduino sends a signal to the BC457 transistor to activate the buzzer, giving an instant warning alert.

- ADXL355 accelerometer

The ADXL335 accelerometer is used to detect vibrations or movements in three directions—X, Y, and Z axes. It converts these physical movements into analog voltage signals, which are then sent to the Arduino. When the accelerometer senses sudden changes in acceleration (like during an earthquake), these signals help the Arduino determine whether the motion is strong enough to trigger the buzzer alarm

- Transistor

The BC457 transistor is used as an electronic switch in the circuit. It is connected between the Arduino and the buzzer. When the Arduino detects an earthquake-like vibration, it sends a small current to the base of the transistor. This allows a larger current to flow from the collector to the emitter, activating the buzzer. Without the transistor, the Arduino might not be able to provide enough current to power the buzzer directly.

- Buzzer

The buzzer serves as the alert system in the project. When the Arduino detects unusual vibrations through the accelerometer and activates the transistor, the buzzer turns on and produces a loud sound. This sound acts as an immediate warning signal to alert people nearby about possible earthquake activity.

- 1K ohm resistor

The 1 kΩ resistor is used to limit the current flowing into the base of the BC457 transistor from the Arduino. This protects both the transistor and the Arduino pin from excess current. It ensures that the transistor switches properly without drawing too much current from the microcontroller.

7.     circuit diagram

✅ Step 1: Powering the Circuit

- Connect VCC of the ADXL335 to the 5V pin of the Arduino.

- Connect GND of the ADXL335 to GND on Arduino.

- This powers the accelerometer.

✅ Step 2: Reading Motion Data

- Connect the X, Y, Z output pins of the ADXL335 to Arduino analog pins (e.g., A0, A1, A2).

- These analog pins will read the acceleration data from the sensor in real time.

✅ Step 3: Motion Detection Logic

- In the Arduino code, calculate the change in acceleration using the X, Y, Z values.

- If a sudden spike is detected (which indicates a tremor), the Arduino will output a HIGH signal to trigger the alert system.

✅ Step 4: Controlling the Buzzer with Transistor

- The buzzer is connected to the collector of the BC457 transistor.

- The emitter of the transistor goes to GND.

- The base of the transistor is connected to a digital pin on the Arduino (e.g., D9) through a 1 kΩ resistor.

✅ Step 5: Role of the 1 kΩ Resistor

- The 1 kΩ resistor limits the current flowing from the Arduino to the base of the transistor.

- This protects the Arduino and the transistor from excessive current.

✅ Step 6: Triggering the Alert

- When the Arduino detects strong movement, it sends a HIGH signal to the transistor’s base.

- This allows current to flow from collector to emitter, activating the buzzer.

8.  programming

int xPin = A0;

int yPin = A1;

int zPin = A2;

int buzzer = 9;

void setup() {

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

int x = analogRead(xPin);

int y = analogRead(yPin);

int z = analogRead(zPin);

// Calculate magnitude (simple method)

float magnitude = sqrt(sq(x - 512) + sq(y - 512) + sq(z - 512));

if (magnitude > 100) { // Threshold value (tweak as needed)

digitalWrite(buzzer, HIGH);

} else {

digitalWrite(buzzer, LOW);

}

delay(100);

}

🧾 Code Explanation

cpp

CopyEdit

int xPin = A0;

int yPin = A1;

int zPin = A2;

- These lines assign the accelerometer output pins (X, Y, Z axes) to analog pins A0, A1, and A2 on the Arduino.

- The Arduino will use these pins to read motion data.

cpp

CopyEdit

int buzzer = 9;

- The buzzer is connected to digital pin 9 on the Arduino. This pin will be used to turn the buzzer ON or OFF.

🔧 setup() function

cpp

CopyEdit

void setup() {

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

- pinMode(buzzer, OUTPUT); tells the Arduino that pin 9 will be used to send an output signal to control the buzzer.

- Serial.begin(9600); starts the Serial Monitor at a speed of 9600 bits per second — useful for debugging or viewing sensor values on your computer.

🔁 loop() function

cpp

CopyEdit

int x = analogRead(xPin);

int y = analogRead(yPin);

int z = analogRead(zPin);

- These lines read analog values (between 0 and 1023) from the accelerometer on each axis (X, Y, Z).

cpp

CopyEdit

float magnitude = sqrt(sq(x - 512) + sq(y - 512) + sq(z - 512));

- This line calculates the overall magnitude of motion using a basic form of the 3D distance formula.

- The value 512 is used as the resting midpoint (for ADXL335, values are near 512 when there's no movement).

- If the sensor experiences vibration or tilt, these values change — resulting in a higher magnitude.

cpp

CopyEdit

if (magnitude > 100) {

digitalWrite(buzzer, HIGH);

} else {

digitalWrite(buzzer, LOW);

}

- If the calculated magnitude is greater than 100 (a threshold you can adjust), it means sudden movement (like an earthquake) is detected.

- So, the buzzer is turned ON (HIGH).

- If no strong motion is detected, the buzzer stays OFF.

cpp

CopyEdit

delay(100);

- A small pause of 100 milliseconds before the next reading — helps reduce noise and unnecessary flickering.

Group members name and registration

- Shreeyansh janu 12419027

- Smit kumar das 12409850

- Mohamed Hassan Abdikarin 12412448

- Kartikey Sengar 12405305